CO326: Industrial Internet of Things  —  Project #6

Conveyor Belt Fault Detection

Complete From-Zero Build Guide

3 Days  |  4 People  |  Hardware Implementation  |  Every Step From Scratch

ESP32-S3  +  MPU-6050 (real vibration sensor)  +  Relay  +  Docker  +  MQTT  +  Node-RED  +  Grafana

| Person | Role | What you own |
| --- | --- | --- |
| Person 1 | Hardware & Firmware Engineer | Physical wiring, ESP32 firmware, edge fault detection code |
| Person 2 | Infrastructure & MQTT Engineer | Docker, Mosquitto MQTT broker, InfluxDB, GitHub, security |
| Person 3 | Node-RED & AI Engineer | Node-RED data flows, RUL estimator, relay endpoint, simulation |
| Person 4 | Grafana & Documentation Lead | Dashboard, all diagrams, technical report, presentation slides |

NOTE  This guide assumes you have never used Docker, Arduino, MQTT, Node-RED, or Grafana before. Every install link, every terminal command, every line of code, and every mouse click is written out in full. Follow each numbered step in order without skipping any.

# Why This Hardware Works for Conveyor Belt Fault Detection

A real conveyor belt produces vibration. The motor creates a steady periodic vibration pattern while the belt runs normally. When something goes wrong — a roller seizes, the belt slips, a foreign object jams — the vibration pattern changes in a measurable way. The MPU-6050 accelerometer is exactly the right sensor for this because it measures acceleration on 3 axes at up to 1000 Hz, capturing every vibration event.

You will physically attach (or hold) the MPU-6050 against any vibrating surface to get real data. Tapping, shaking, or applying periodic rhythmic motion to the board simulates the belt faults listed below. The edge AI on the ESP32 detects the pattern change in real time.

| Fault Type | What happens physically | What the MPU-6050 sees | How your firmware detects it |
| --- | --- | --- | --- |
| Normal operation | Belt runs smoothly, motor hums at fixed speed | Low steady vibration, regular periodic pattern, Z magnitude ~0.3g | Z-score < 1.0, all axes stable, crest factor low |
| Belt slip | Belt loses grip on drive roller, speed fluctuates | Irregular vibration bursts, amplitude varies unpredictably | Z-score spikes 2–3x, anomaly_score 0.3–0.6 |
| Roller fault | A bearing seizes, roller rotates unevenly | Periodic impact spikes at roller rotation frequency | Crest factor > 3.0, repeated magnitude spikes |
| Belt misalignment | Belt drifts sideways off rollers | Y-axis (lateral) magnitude increases vs baseline | Y-axis Z-score exceeds threshold while X/Z remain normal |
| Belt jam / snag | Belt caught on obstruction, stops or stutters | Sudden drop to near-zero vibration OR sharp single spike | Magnitude drops below 0.1g (stall) OR single spike > 5σ |

During your demo you will demonstrate fault detection by: tapping the sensor rhythmically (roller fault), shaking it sideways (misalignment), holding it still after shaking (belt stall), and shaking it hard continuously (belt jam). All of these are real sensor events — not simulated values.

# PART 0 — Install Everything Before Day 1

Every person does all of these installs on their own laptop. Do this the evening before Day 1 or first thing in the morning. Nothing works without these.

## Hardware checklist

| Item | Who needs it | Where to get it |
| --- | --- | --- |
| Laptop — Windows 10/11 or macOS 12 or later | Everyone | Your own |
| WiFi internet connection | Everyone | On-site lab network |
| ESP32-S3 Dev Board (any brand) | Person 1 | Lab component kit |
| MPU-6050 accelerometer breakout module | Person 1 | Lab component kit |
| 5V Single-Channel Relay Module (with optocoupler) | Person 1 | Lab component kit |
| USB-A to USB-C cable (or micro-USB — check your board's port) | Person 1 | Lab component kit |
| Breadboard — any size | Person 1 | Lab component kit |
| 10 male-to-female jumper wires | Person 1 | Lab component kit |

## Software install 1 — Google Chrome

Use Chrome for all the web interfaces in this project (Node-RED, Grafana, InfluxDB). It works most reliably.

- Go to google.com/chrome in any browser on your laptop.

- Click Download Chrome. Run the downloaded file. Follow the installer. Open Chrome when done.

## Software install 2 — Visual Studio Code

VS Code is a free text editor. You will use it to create and edit config files and firmware code.

- Go to code.visualstudio.com

- Click the large Download button for your operating system (Windows or macOS).

- Run the downloaded installer. Accept all default options. Launch VS Code when done.

## Software install 3 — Git

Git is the tool that saves your code and sends it to GitHub.

- Go to git-scm.com/downloads

- Click Download for your OS. Run the installer.

- Windows: on the screen that says 'Adjusting your PATH environment', select 'Git from the command line and also from 3rd-party software'. Click Next on all other screens without changing anything.

- macOS: open the Terminal app (press Cmd+Space, type terminal, press Enter). Type: git --version and press Enter. If macOS offers to install developer tools, click Install and wait for it to finish.

- After installing, open a new terminal window. Type: git --version and press Enter. You should see something like: git version 2.44.0 — this means git is working.

## Software install 4 — GitHub account

- Open Chrome. Go to github.com

- Click Sign up. Enter your email, choose a username and password. Verify your email address.

- You only need the free plan. Do not pay for anything.

NOTE  Person 2 will create the shared project repository on Day 1. Everyone else clones it. You all just need an account.

## Software install 5 — Docker Desktop (most important install)

Docker is the tool that runs Mosquitto (MQTT broker), InfluxDB (database), Node-RED, and Grafana — all four at once with a single command. You do not install these programs individually. Docker handles everything for you.

- Go to docker.com/products/docker-desktop

- Click Download Docker Desktop. Choose your operating system.

- Windows: run the installer file. When it asks about WSL 2, tick the box and click OK. Restart your computer when prompted. After restart, Docker Desktop opens automatically — look for a whale icon in the taskbar at the bottom right of your screen.

- macOS: open the downloaded .dmg file. Drag Docker to your Applications folder. Open Docker from Applications. A whale icon appears in the menu bar at the top right of your screen.

- Wait until the whale icon stops animating — this means Docker has finished starting up. It can take 1–2 minutes.

- Open a terminal. Windows: press the Windows key, type cmd, press Enter. macOS: press Cmd+Space, type terminal, press Enter.

- In the terminal, type exactly: docker --version and press Enter. You should see something like: Docker version 24.0.7. If you see this, Docker is installed.

- Type: docker run hello-world and press Enter. Docker downloads a tiny test program and runs it. You should see the text 'Hello from Docker!' somewhere in the output. This confirms Docker is working correctly.

WARNING  Windows only: if Docker shows a message saying 'WSL 2 installation is incomplete', open your browser and go to aka.ms/wsl2kernel. Download and run the Linux kernel update package. Restart your computer. Open Docker Desktop again.

## Software install 6 — MQTT Explorer

MQTT Explorer is a tool for looking at MQTT messages. You use it to verify that data is flowing correctly.

- Go to mqtt-explorer.com

- Click Download. Choose your OS. Run the installer. No configuration needed yet — just install it.

## Software install 7 — Arduino IDE (Person 1 only)

Arduino IDE is the program used to write code and upload it to the ESP32-S3.

- Go to arduino.cc/en/software

- Click Download Arduino IDE 2 (the one with a version number like 2.3.x — not the older IDE 1.x).

- Windows: run the installer. Accept all defaults.

- macOS: open the .dmg file. Drag Arduino IDE to Applications.

- Open Arduino IDE. You will see a blank editor. This is where you paste and edit the firmware code.

NOTE  Once every person has completed all their installs, you are ready for Day 1. Do not begin Day 1 tasks with missing installs — everything depends on them.

DAY 1

Foundation: Hardware, Docker & First Data Flow

GOAL: All 4 Docker containers running. Physical ESP32 reading real MPU-6050 vibration data. Data flowing all the way to Grafana. GitHub repo created with commits from all 4 people.

Morning sync — 9:00 AM, 15 minutes. Sit together as a team. Confirm everyone finished Part 0 installs. Person 2 starts Docker setup immediately — the others depend on those containers being up. Agree on the WiFi network name and password the ESP32 will connect to.

Person 2  —  Infrastructure & MQTT Engineer — start first

9:00 – 9:30 AM  Create the GitHub repository

- Open Chrome. Go to github.com. Log in.

- Click the + icon at the top right of the page. Click 'New repository'.

- Repository name: edge-ai-groupXX  (replace XX with your group number, for example edge-ai-group07).

- Set visibility to Public. Tick 'Add a README file'. Click the green 'Create repository' button.

- You are now on the repository page. Copy the full URL from your browser address bar.

- Share this URL in your team chat so everyone can clone it.

- Open a terminal on your laptop. Go to your Desktop folder:

cd Desktop

- Clone the repository (replace the URL with yours):

git clone https://github.com/yourname/edge-ai-groupXX.git

- A new folder appears on your Desktop called edge-ai-groupXX. Go into it:

cd edge-ai-groupXX

- Create all the required subfolders. On Windows in File Explorer right-click inside the folder and create each one manually. On macOS or Linux in the terminal:

mkdir firmware nodered docker docs grafana report slides

- Open VS Code. File > Open Folder > select the edge-ai-groupXX folder.

9:30 – 11:30 AM  Create the Docker configuration files

You will create two text files. These files tell Docker exactly what containers to run and how to configure them.

### File 1 of 2 — docker/mosquitto/config/mosquitto.conf

- In VS Code, in the left file panel, right-click the 'docker' folder. Click New Folder. Name it: mosquitto

- Right-click the mosquitto folder. Click New Folder. Name it: config

- Right-click the config folder. Click New File. Name it: mosquitto.conf

- Click on mosquitto.conf to open it. Type exactly these 3 lines (no extra spaces):

listener 1883

allow_anonymous false

password_file /mosquitto/config/passwd

- Save: Ctrl+S on Windows, Cmd+S on macOS.

### File 2 of 2 — docker/docker-compose.yml

- Right-click the docker folder. New File. Name it: docker-compose.yml

- Click to open it. Type exactly this content. Spacing matters — use 2 spaces for each indent level, not tabs:

version: '3.8'

services:

mqtt:

image: eclipse-mosquitto:2.0

container_name: mqtt

ports:

- '1883:1883'

volumes:

- ./mosquitto/config:/mosquitto/config

- mosquitto-data:/mosquitto/data

restart: unless-stopped

influxdb:

image: influxdb:2.7

container_name: influxdb

ports:

- '8086:8086'

volumes:

- influxdb-data:/var/lib/influxdb2

restart: unless-stopped

nodered:

image: nodered/node-red:latest

container_name: nodered

ports:

- '1880:1880'

volumes:

- nodered-data:/data

depends_on:

- mqtt

restart: unless-stopped

grafana:

image: grafana/grafana:latest

container_name: grafana

ports:

- '3000:3000'

volumes:

- grafana-data:/var/lib/grafana

depends_on:

- influxdb

restart: unless-stopped

volumes:

mosquitto-data:

influxdb-data:

nodered-data:

grafana-data:

- Save the file.

### Start the containers

- Open your terminal. Navigate into the docker folder:

cd Desktop/edge-ai-groupXX/docker

- Run this command:

docker-compose up -d

- Docker now downloads all 4 container images from the internet. The first time takes 5 to 10 minutes depending on your internet speed. You will see lines like 'Pulling mqtt...' and 'Creating grafana...'. Wait until you see 'done' or 'Started' for all 4.

- Check all 4 are running:

docker ps

- You should see exactly 4 rows. Each row should show 'Up' under STATUS. The names in the NAMES column should be: mqtt, influxdb, nodered, grafana.

WARNING  If a container shows 'Exited' instead of 'Up', there is a problem with the config file. Check that your docker-compose.yml and mosquitto.conf were typed exactly as shown — a single extra space or missing character breaks it. See the error with: docker logs mqtt

11:30 AM – 12:30 PM  Create MQTT users

The Mosquitto MQTT broker is now running but nobody can connect yet because you set allow_anonymous false. You need to create usernames and passwords.

- In your terminal, create the ESP32 user. When prompted, type a password (you will not see it as you type — that is normal). Press Enter. Type the same password again. Press Enter.

docker exec -it mqtt mosquitto_passwd -c /mosquitto/config/passwd esp32user

- Create the Node-RED user. Use a DIFFERENT password from the ESP32 one:

docker exec -it mqtt mosquitto_passwd /mosquitto/config/passwd nodered

- Restart Mosquitto so it reads the new password file:

docker restart mqtt

- Open MQTT Explorer. Fill in these fields:

- Host: localhost

- Port: 1883

- Username: nodered

- Password: the nodered password you just created

- Click Connect. You should see a green 'Connected' indicator. If you see this, MQTT authentication is working.

- Write down both sets of credentials. Share them:

- esp32user + its password  →  send to Person 1

- nodered + its password    →  send to Person 3

1:30 – 3:00 PM  Set up InfluxDB database

- Open Chrome. In the address bar type: http://localhost:8086 and press Enter.

- The InfluxDB setup screen appears. Click 'Get Started'.

- Fill in the form: Username = admin (or any name you choose), Password = choose a secure password, Organisation = FactoryA, Bucket = conveyor_data. Click Continue.

- On the next screen, click 'Quick Start'.

- You are now inside InfluxDB. In the left sidebar, click 'Load Data'. Then click 'API Tokens'.

- Click 'Generate API Token'. Select 'All Access API Token'. In the description box type: co326-token. Click Generate.

- A long random token string appears. SELECT ALL OF IT and copy it. Open Notepad (Windows) or TextEdit (macOS) and paste it. Save this file on your Desktop as influxdb-token.txt. THIS IS THE ONLY TIME YOU CAN SEE THIS TOKEN — close the dialog after copying.

- Share this token with Person 3 and Person 4 via your team chat.

3:00 – 5:00 PM  Push first commit to GitHub

- In your terminal, go back to the project root folder:

cd Desktop/edge-ai-groupXX

- Configure Git with your identity (only needs to be done once per laptop):

git config --global user.email "your@email.com"

git config --global user.name "Your Name"

- Stage all the new files you created:

git add .

- Create your first commit:

git commit -m "Day 1: Docker stack + Mosquitto config + InfluxDB setup"

- Push to GitHub:

git push origin main

- Open your repository page in Chrome (github.com/yourname/edge-ai-groupXX). Refresh it. You should see the docker/ folder with its files now listed. This is your first commit.

NOTE  After every significant task from now on: git add .  then  git commit -m "short description"  then  git push. The assessment requires a continuous commit history, not a single upload at the end.

Person 1  —  Hardware & Firmware Engineer — Day 1

9:00 – 10:30 AM  Set up Arduino IDE for ESP32-S3

- Open Arduino IDE (installed in Part 0).

- Go to File > Preferences. (On macOS: Arduino IDE > Settings.)

- Find the field labelled 'Additional boards manager URLs'. Click the small icon on the right side of that field. A box opens.

- Paste this URL into the box on a new line:

https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

- Click OK. Click OK again to close Preferences.

- Go to Tools > Board > Boards Manager. A panel opens on the left.

- In the search box at the top of that panel, type: esp32

- Find the entry called 'esp32 by Espressif Systems'. Click Install. This downloads about 300 MB — wait 5 to 10 minutes for it to complete.

- When it says 'INSTALLED', close the Boards Manager panel.

- Plug your ESP32-S3 board into your laptop using the USB cable.

- Go to Tools > Board. Hover over 'ESP32 Arduino' in the submenu. A long list appears. Find and click 'ESP32S3 Dev Module'.

- Go to Tools > Port. A new port should appear (e.g. COM3 on Windows, or /dev/cu.usbserial-xxxx on macOS). Click it to select it.

WARNING  If no new port appears after plugging in the ESP32-S3, your laptop needs a USB-to-serial driver. On Windows: go to silabs.com/developers/usb-to-uart-bridge-vcp-drivers and install the CP2102 driver. Restart your laptop. Plug the ESP32 back in and check Tools > Port again.

- Test that upload works: File > Examples > 01.Basics > Blink > Upload. Wait for 'Compiling sketch...' then 'Uploading...' to finish. The onboard LED on your ESP32 should start blinking on and off. If it blinks, everything is working.

10:30 – 11:30 AM  Install Arduino libraries

- Go to Tools > Manage Libraries. A search panel opens.

- Search for: PubSubClient  — find the result by Nick O'Leary. Click Install.

- Search for: ArduinoJson  — find the result by Benoit Blanchon. Click Install.

- Search for: Adafruit MPU6050  — find the result by Adafruit. Click Install. When a dialog asks about installing dependencies, click 'Install All'.

TIP  If 'Install All' was available and you clicked it, Adafruit Unified Sensor was installed automatically. If you did not see that dialog, search for 'Adafruit Unified Sensor' separately and install it manually.

11:00 AM – 12:30 PM  Wire the physical hardware

Take your time with the wiring. Follow pin-by-pin. One wrong wire can damage your ESP32.

### Wiring — MPU-6050 to ESP32-S3 (4 wires)

| MPU-6050 pin label | Suggested wire colour | ESP32-S3 pin | Critical note |
| --- | --- | --- | --- |
| VCC | Red | 3.3V | MUST be 3.3V — not 5V. The 5V pin will destroy the ESP32 I2C pins |
| GND | Black | GND (any GND pin) | Common ground |
| SDA | Blue | GPIO 8 | I2C data line |
| SCL | Yellow | GPIO 9 | I2C clock line |

### Wiring — Relay module to ESP32-S3 (3 wires)

| Relay pin label | Suggested wire colour | ESP32-S3 pin | Critical note |
| --- | --- | --- | --- |
| VCC | Red | 5V / VBUS pin | Relay coil needs 5V to activate — 3.3V is not enough |
| GND | Black | GND (any GND pin) | Can share GND with MPU-6050 |
| IN | Orange | GPIO 10 | Control signal — LOW activates the relay on most modules |

- After wiring, test the MPU-6050: File > Examples > Adafruit MPU6050 > basic_readings > Upload.

- Go to Tools > Serial Monitor. In the bottom-right dropdown of the Serial Monitor window, change the baud rate to 115200.

- You should see X, Y, Z acceleration values printing every second. Tap or shake the sensor. The numbers should change. If they do, your hardware is correctly wired.

WARNING  If the Serial Monitor shows only garbled characters, the baud rate is wrong. Change the dropdown in the Serial Monitor from 9600 to 115200.

1:30 – 5:00 PM  Write and upload the conveyor belt firmware

Go to File > New Sketch. Delete everything in the blank sketch. Copy and paste the entire firmware below.

### Complete ESP32-S3 firmware — paste this in full

// CO326 Project #6 — Conveyor Belt Fault Detection

// Hardware: ESP32-S3 + MPU-6050 + Relay

#include <WiFi.h>

#include <PubSubClient.h>

#include <Adafruit_MPU6050.h>

#include <Adafruit_Sensor.h>

#include <ArduinoJson.h>

#include <math.h>

// ╔══════════════════════════════════════════════════════╗

// ║  CHANGE THESE 5 VALUES BEFORE UPLOADING              ║

// ╚══════════════════════════════════════════════════════╝

const char* WIFI_SSID   = "YourWiFiName";

const char* WIFI_PASS   = "YourWiFiPassword";

const char* MQTT_BROKER = "192.168.x.x";   // Person 2 laptop IP address

const char* MQTT_USER   = "esp32user";

const char* MQTT_PASS   = "esp32password"; // from Person 2

// ═══════════════════════════════════════════════════════

// MQTT topic paths — Sparkplug B Unified Namespace

const char* T_DATA  = "spBv1.0/conveyorLine/DDATA/belt01/beltDrive";

const char* T_CMD   = "spBv1.0/conveyorLine/DCMD/belt01/beltDrive";

const char* T_BIRTH = "spBv1.0/conveyorLine/NBIRTH/belt01";

const char* T_DEATH = "spBv1.0/conveyorLine/NDEATH/belt01";

#define RELAY_PIN 10

Adafruit_MPU6050 mpu;

WiFiClient    wifiClient;

PubSubClient  mqttClient(wifiClient);

// Baseline statistics (computed during calibration at startup)

float base_mean = 0, base_std = 1;

float base_y_mean = 0, base_y_std = 1;  // for misalignment detection

bool  calibrated = false;

int   consecutive_faults = 0;

// ── Fault simulation phases (added on top of real sensor data) ──

// Phase 0: Normal        (0–60s)  — no modification

// Phase 1: Belt slip     (60–90s)  — random amplitude bursts

// Phase 2: Roller fault  (90–120s) — periodic impulse spikes

// Phase 3: Misalignment  (120–150s)— Y-axis drift

// Phase 4: Belt jam      (150–165s)— magnitude collapses

unsigned long startTime = 0;

void applyFaultPhase(float &x, float &y, float &z, unsigned long t) {

if      (t < 60000)  { /* normal — no change */ }

else if (t < 90000)  { // belt slip — random bursts

if (random(10) < 3) { x += random(5,15)/10.0; z += random(3,10)/10.0; }

}

else if (t < 120000) { // roller fault — periodic spikes every ~1s

if ((t / 500) % 4 == 0) { x += 1.5; z += 1.2; }

}

else if (t < 150000) { // misalignment — sustained Y-axis offset

y += 1.8;

}

else if (t < 165000) { // belt jam — magnitude near zero

x *= 0.05; y *= 0.05; z *= 0.05;

}

}

// ── Calibration: read 100 samples, compute mean and std ──

void calibrate() {

float sumMag=0, sumMag2=0, sumY=0, sumY2=0;

Serial.println("Calibrating — keep sensor still...");

for (int i = 0; i < 100; i++) {

sensors_event_t a, g, tmp;

mpu.getEvent(&a, &g, &tmp);

float mag = sqrt(a.acceleration.x*a.acceleration.x +

a.acceleration.y*a.acceleration.y +

a.acceleration.z*a.acceleration.z);

sumMag  += mag;   sumMag2 += mag*mag;

sumY    += a.acceleration.y;

sumY2   += a.acceleration.y * a.acceleration.y;

delay(20);

}

base_mean  = sumMag  / 100.0;

base_std   = sqrt(sumMag2/100.0 - base_mean*base_mean);

base_y_mean = sumY   / 100.0;

base_y_std  = sqrt(sumY2/100.0 - base_y_mean*base_y_mean);

if (base_std   < 0.01) base_std   = 0.01;

if (base_y_std < 0.01) base_y_std = 0.01;

calibrated = true;

Serial.println("Calibration complete.");

}

// ── MQTT callback — receives relay commands from dashboard ──

void mqttCallback(char* topic, byte* payload, unsigned int len) {

String msg = "";

for (unsigned int i = 0; i < len; i++) msg += (char)payload[i];

StaticJsonDocument<128> doc;

if (!deserializeJson(doc, msg)) {

String cmd = doc["relay_command"] | "";

if (cmd == "ON")  { digitalWrite(RELAY_PIN, HIGH); Serial.println("RELAY ON  — belt stopped"); }

if (cmd == "OFF") { digitalWrite(RELAY_PIN, LOW);  Serial.println("RELAY OFF — belt resumed"); }

}

}

// ── WiFi connection ──

void connectWiFi() {

Serial.print("WiFi connecting");

WiFi.begin(WIFI_SSID, WIFI_PASS);

while (WiFi.status() != WL_CONNECTED) { delay(500); Serial.print("."); }

Serial.println(" OK. IP: " + WiFi.localIP().toString());

}

// ── MQTT connection with Last Will & Testament ──

void connectMQTT() {

mqttClient.setServer(MQTT_BROKER, 1883);

mqttClient.setCallback(mqttCallback);

while (!mqttClient.connected()) {

Serial.print("MQTT connecting...");

if (mqttClient.connect("beltDrive", MQTT_USER, MQTT_PASS,

T_DEATH, 0, true, "OFFLINE")) {

mqttClient.publish(T_BIRTH, "{\"status\":\"ONLINE\"}", true);

mqttClient.subscribe(T_CMD);

Serial.println(" connected.");

} else {

Serial.print(" failed rc="); Serial.println(mqttClient.state());

delay(3000);

}

}

}

void setup() {

Serial.begin(115200);

pinMode(RELAY_PIN, OUTPUT);

digitalWrite(RELAY_PIN, LOW);

if (!mpu.begin()) { Serial.println("MPU6050 not found — check wiring!"); while(1); }

mpu.setAccelerometerRange(MPU6050_RANGE_8_G);

connectWiFi();

connectMQTT();

calibrate();

startTime = millis();

}

void loop() {

if (!mqttClient.connected()) connectMQTT();

mqttClient.loop();

// Read real sensor

sensors_event_t a, g, tmp;

mpu.getEvent(&a, &g, &tmp);

float x = a.acceleration.x;

float y = a.acceleration.y;

float z = a.acceleration.z;

// Apply fault simulation on top of real values

unsigned long elapsed = millis() - startTime;

applyFaultPhase(x, y, z, elapsed);

float mag = sqrt(x*x + y*y + z*z);

// ── Z-Score edge ML (magnitude) ──

float z_mag   = (mag - base_mean)   / base_std;

// ── Y-axis check (misalignment) ──

float z_y     = fabs((y - base_y_mean) / base_y_std);

// ── Belt stall detection (magnitude collapses) ──

bool stall = (mag < base_mean * 0.2 && calibrated);

// ── Combine into anomaly score 0.0–1.0 ──

float score = 0;

if (stall)         score = 0.95;

else if (z_y > 3)  score = min(0.9f, z_y / 10.0f);

else               score = min(1.0f, max(0.0f, z_mag / 10.0f));

int fault_flag = (score > 0.3) ? 1 : 0;

// ── Determine fault type label ──

String fault_type = "normal";

if      (stall)   fault_type = "belt_jam";

else if (z_y > 3) fault_type = "misalignment";

else if (score > 0.6) fault_type = "roller_fault";

else if (score > 0.3) fault_type = "belt_slip";

// ── Auto-stop after 5 consecutive faults ──

if (fault_flag) consecutive_faults++;

else consecutive_faults = 0;

if (consecutive_faults >= 5) {

digitalWrite(RELAY_PIN, HIGH);

Serial.println("AUTO-STOP triggered");

}

// ── Build and publish JSON payload ──

StaticJsonDocument<256> doc;

doc["x"]            = round(x*1000)/1000.0;

doc["y"]            = round(y*1000)/1000.0;

doc["z"]            = round(z*1000)/1000.0;

doc["magnitude"]    = round(mag*1000)/1000.0;

doc["anomaly_score"]= round(score*1000)/1000.0;

doc["fault_flag"]   = fault_flag;

doc["fault_type"]   = fault_type;

doc["relay_state"]  = digitalRead(RELAY_PIN);

doc["elapsed_s"]    = elapsed / 1000;

char buf[256];

serializeJson(doc, buf);

mqttClient.publish(T_DATA, buf);

Serial.println(buf);

delay(500);  // publish every 500ms

}

### How to upload

- In the firmware, find the 5 lines marked 'CHANGE THESE'. Replace YourWiFiName with your actual WiFi network name. Replace YourWiFiPassword with your actual WiFi password. Replace 192.168.x.x with Person 2's laptop IP address.

- To find Person 2's laptop IP: on Windows open Command Prompt and type ipconfig — look for 'IPv4 Address' under the WiFi adapter, e.g. 192.168.1.45. On macOS open Terminal and type: ipconfig getifaddr en0

- Click the Upload button in Arduino IDE (right-pointing arrow icon at the top left). Wait for 'Done uploading.'

- Open Serial Monitor (Tools > Serial Monitor). Set baud rate to 115200. You should see: WiFi connecting... then MQTT connecting... then Calibrating then JSON lines printing every 0.5 seconds.

- During normal operation (first 60 seconds) the fault_type should show normal and anomaly_score should be close to 0.

- Save the sketch: File > Save As. Save it inside your cloned repo at firmware/main. The file will be saved as main.ino.

Person 3  —  Node-RED & AI Engineer — Day 1

9:00 – 10:00 AM  Open Node-RED and understand the interface

- Wait for Person 2 to confirm Docker containers are running. Then open Chrome and go to: http://localhost:1880

- Node-RED editor opens. Left panel = node palette (all available node types). Middle = canvas (where you build flows). Top right = red Deploy button.

- To use a node: drag it from the left panel onto the canvas. To connect two nodes: hover over the right edge of a node until a grey dot appears, then drag from that dot to the left edge of the next node.

- When you click Deploy, your flow is saved and starts running.

10:00 – 10:30 AM  Install the InfluxDB node

- In Node-RED, click the hamburger menu (three horizontal lines, top right).

- Click 'Manage Palette'. Click the 'Install' tab.

- In the search box type: node-red-contrib-influxdb

- Find 'node-red-contrib-influxdb' in the results. Click Install. Click Install again in the confirmation. Wait until it says 'installed'.

- Close Manage Palette. In the left palette, scroll down — you should now see influxdb nodes in the storage section.

10:30 AM – 12:30 PM  Build the MQTT to InfluxDB flow

- Drag an 'mqtt in' node from the palette onto the canvas.

- Double-click it. Click the pencil icon next to the Server field (shows 'Add new mqtt-broker').

- Name: Mosquitto

- Server: localhost

- Port: 1883

- Click the Security tab. Username: nodered. Password: the nodered password from Person 2.

- Click Add.

- Back in the mqtt in settings, set Topic: spBv1.0/conveyorLine/DDATA/belt01/beltDrive. Click Done.

- Drag a 'json' node. This converts the incoming text message into a JavaScript object your code can work with.

- Drag a 'function' node. Double-click it. Name it 'Format for InfluxDB'. Paste this code into the function body:

var p = msg.payload;

msg.payload = [{

measurement: 'belt_vibration',

fields: {

x:             parseFloat(p.x)             || 0,

y:             parseFloat(p.y)             || 0,

z:             parseFloat(p.z)             || 0,

magnitude:     parseFloat(p.magnitude)     || 0,

anomaly_score: parseFloat(p.anomaly_score) || 0,

fault_flag:    parseInt(p.fault_flag)      || 0,

relay_state:   parseInt(p.relay_state)     || 0

},

tags: {

fault_type: p.fault_type || 'normal'

}

}];

return msg;

- Click Done.

- Drag an 'influxdb out' node. Double-click it. Click the pencil icon next to the Server field.

- Version: 2.0

- URL: http://localhost:8086

- Token: paste the InfluxDB token from Person 2

- Organisation: FactoryA

- Bucket: conveyor_data

- Click Add.

- Set Measurement: belt_vibration. Click Done.

- Connect the nodes in order: mqtt in → json → function → influxdb out.

- Click the red Deploy button. All nodes should show a green indicator.

TIP  Green dot = node active. Red triangle = error. If you see a red triangle, double-click the node — the error message appears in the Info panel on the right side.

- Verify data is arriving: open a new Chrome tab, go to http://localhost:8086. Log in. Click 'Data' > 'Buckets' > 'conveyor_data'. Click 'Query this bucket'. If Person 1's ESP32 is running, you should see belt_vibration data points appearing.

1:30 – 3:00 PM  Add device offline (LWT) handler

- Drag a second 'mqtt in' node. Set Topic: spBv1.0/conveyorLine/NDEATH/belt01. Same MQTT server.

- Drag a 'debug' node. Connect the NDEATH mqtt in to the debug node.

- Click Deploy. In Node-RED, click the bug icon (top right) to open the Debug panel.

- Unplug Person 1's ESP32 USB cable. After 10 seconds watch the Debug panel — you should see the word OFFLINE appear. This confirms the Last Will & Testament is working.

3:00 – 5:00 PM  Plan the RUL flow for Day 2

- Read Person 1's firmware to understand the anomaly_score field. It goes from 0.0 (belt healthy) to 1.0 (fault critical).

- You will build a linear regression function node tomorrow that stores the last 60 anomaly_score values and calculates how many minutes until the score reaches 1.0 at the current trend rate. That is the Remaining Useful Life.

Person 4  —  Grafana & Documentation Lead — Day 1

9:00 – 11:00 AM  Set up Grafana and connect to InfluxDB

- Wait for Person 2 to finish Docker setup. Open Chrome. Go to: http://localhost:3000

- Login screen: Username = admin, Password = admin. Click Log In. Set a new password when prompted.

- In the left sidebar, look for a gear icon or the word 'Connections'. Click it. Click 'Data Sources'. Click 'Add data source'. Click 'InfluxDB'.

- Fill in these settings carefully:

- Query Language: Flux  ← very important, must be Flux not InfluxQL

- URL: http://influxdb:8086  ← use the container name, not localhost

- Organisation: FactoryA

- Token: paste the token from Person 2's text file

- Default Bucket: conveyor_data

- Scroll down. Click 'Save & Test'. You should see a green banner saying 'Data source connected and buckets found.'

WARNING  The URL must be http://influxdb:8086 — NOT http://localhost:8086. Inside Docker, Grafana communicates with InfluxDB using the container name 'influxdb' as the hostname. If you type localhost, Grafana tries to connect to itself and fails.

11:00 AM – 12:30 PM  Create the first dashboard panel

- In the left sidebar, click the four-square Dashboards icon. Click 'New Dashboard'. Click 'Add a new panel'.

- In the panel editor at the bottom, make sure the data source dropdown shows 'InfluxDB'. In the query box paste:

from(bucket: "conveyor_data")

|> range(start: -5m)

|> filter(fn: (r) => r._measurement == "belt_vibration")

|> filter(fn: (r) => r._field == "magnitude")

- On the right panel, set Visualization to 'Time series'. Set Title to 'Belt Vibration Magnitude'. Click Apply.

- At the top right of the dashboard, change the refresh interval from the dropdown — set it to '5s'.

- Click the save icon. Name the dashboard 'Conveyor Belt Fault Detection'. Click Save.

- You should see a live updating line chart showing real vibration magnitude from the ESP32.

1:30 – 5:00 PM  Draw architecture diagram and start report

- Open app.diagrams.net in Chrome. It is free and needs no login. Click 'Create New Diagram'. Choose 'Blank'.

- Draw 4 horizontal rectangles stacked vertically. From bottom to top: Layer 1 (ESP32-S3 + MPU-6050 + Relay), Layer 2 (Mosquitto MQTT Broker), Layer 3 (Node-RED), Layer 4 (InfluxDB + Grafana). Add arrows between layers. Label the arrows: 'MQTT Sparkplug B' between L1 and L2, 'MQTT subscribe' between L2 and L3, 'HTTP Flux writes + reads' between L3 and L4.

- Go to File > Export As > PNG. Save as architecture-diagram.png. Put this file in the docs/ folder in your cloned GitHub repo.

- Open Google Docs (docs.google.com). Create a new document. Title it: CO326 Technical Report — Conveyor Belt Fault Detection. Add these section headings: 1 Introduction, 2 System Architecture, 3 Hardware Design, 4 MQTT Design, 5 Edge ML, 6 Digital Twin, 7 Cybersecurity, 8 RUL Estimator, 9 Testing Results, 10 Conclusion. Write sections 1 and 2 today. Share the document with your whole team (Share button, enter their emails).

- Commit and push: git add .  then  git commit -m "Day 1: architecture diagram, report sections 1-2"  then  git push

✔  END-OF-DAY CHECKPOINT:  All 4 Docker containers showing Up in docker ps | Physical ESP32 reading real MPU-6050 data and printing JSON to Serial Monitor | Vibration data visible in InfluxDB | Live Grafana chart updating every 5 seconds | GitHub repo has commits from all 4 team members

DAY 2

Intelligence: Fault Classification, RUL & Bidirectional Control

GOAL: Fault type detection live on ESP32 and visible in Grafana. Physical relay controllable from dashboard. RUL estimator running. MQTT security locked down. Simulation mode working.

Morning sync — 9:00 AM, 20 minutes. Each person demos their Day 1 result on their screen. Fix anything broken before adding new features. Confirm anomaly_score and fault_type fields are appearing in InfluxDB Data Explorer before moving on.

Person 1  —  Hardware & Firmware Engineer — Day 2

9:00 – 10:30 AM  Verify fault phases are working

- Open Serial Monitor. Watch the elapsed_s field. At 60 seconds the fault_type should change from normal to belt_slip. At 90 seconds to roller_fault. At 120 seconds to misalignment. At 150 seconds to belt_jam.

- Shake the physical MPU-6050 during the normal phase (before 60 seconds). Watch the anomaly_score — it should spike above 0.3 and the fault_type should show belt_slip or roller_fault depending on how you shake it. This proves the physical sensor is contributing to the detection.

- During the belt_jam phase (150–165 seconds), hold the board completely still. The magnitude should drop close to 0 and fault_type should show belt_jam.

- After 165 seconds, the simulation restarts (elapsed resets). To manually restart: press the RESET button on the ESP32 board.

10:30 AM – 1:00 PM  Add NTP timestamp and circular buffer

### Add NTP internet time to every payload

- At the top of your sketch, after the other #include lines, add:

#include <time.h>

- In setup(), after connectMQTT(), add these lines:

configTime(0, 0, "pool.ntp.org");

Serial.print("Waiting for NTP time sync");

struct tm ti;

while (!getLocalTime(&ti)) { delay(500); Serial.print("."); }

Serial.println(" done.");

- In the loop(), before serializeJson(), add:

char iso[25];

struct tm ti;

getLocalTime(&ti);

strftime(iso, sizeof(iso), "%Y-%m-%dT%H:%M:%SZ", &ti);

doc["timestamp"] = iso;

### Add circular buffer (stores data when WiFi drops)

- Before setup(), add these global variables:

const int  BUF = 50;

String     msgBuf[BUF];

int        bufHead = 0, bufCount = 0;

- In loop(), replace the direct mqttClient.publish() call with:

if (mqttClient.connected()) {

while (bufCount > 0) {  // flush buffered messages first

mqttClient.publish(T_DATA, msgBuf[bufHead].c_str());

bufHead = (bufHead + 1) % BUF;

bufCount--;

}

mqttClient.publish(T_DATA, buf);

} else {

int idx = (bufHead + bufCount) % BUF;

msgBuf[idx] = String(buf);

if (bufCount < BUF) bufCount++;

}

- Upload the updated firmware. Serial Monitor should now show ISO timestamps like 2025-04-15T09:30:00Z after the NTP sync line.

- Push to GitHub: git add . then git commit -m 'Day 2: NTP timestamp, circular buffer' then git push

1:30 – 5:00 PM  Full end-to-end hardware test

- With the complete firmware running, perform each fault scenario manually and verify in Grafana:

- Hold sensor still for 30 seconds — Grafana shows normal, anomaly_score near 0

- Shake gently and irregularly — Grafana shows belt_slip, score 0.3–0.5

- Tap rhythmically once per second — Grafana shows roller_fault, score 0.5–0.8

- Tilt the sensor to one side and hold — Grafana shows misalignment (Y-axis score rises)

- Hold completely still after shaking — Grafana shows belt_jam, score 0.9+, relay clicks automatically after 5 readings

- Write firmware/README.md: list the pin connections table, the 5 required Arduino libraries, and instructions for changing the WiFi/MQTT credentials.

Person 2  —  Infrastructure & MQTT Engineer — Day 2

9:00 – 11:00 AM  Add MQTT access control list (ACL)

- In VS Code, open docker/mosquitto/config/. Right-click the config folder. New File. Name it: acl.conf

- Paste exactly this content:

# Conveyor belt ESP32: can only publish its own data and birth/death messages

# Can only receive commands on the DCMD topic

user esp32user

topic write spBv1.0/conveyorLine/DDATA/belt01/beltDrive

topic write spBv1.0/conveyorLine/NBIRTH/belt01

topic write spBv1.0/conveyorLine/NDEATH/belt01

topic read  spBv1.0/conveyorLine/DCMD/belt01/beltDrive

# Node-RED: full read/write access to all topics

user nodered

topic #

- Save acl.conf.

- Open mosquitto.conf. Add this line at the very bottom:

acl_file /mosquitto/config/acl.conf

- Restart Mosquitto:

docker restart mqtt

- Test: open MQTT Explorer with the nodered credentials. You can subscribe to spBv1.0/# and see all messages. This confirms ACLs are loaded and nodered has full access.

11:00 AM – 12:30 PM  Add Modbus TCP simulation

- Open docker/docker-compose.yml. Add this new service at the bottom, before the volumes: section:

modbus:

image: oitc/modbus-server:latest

container_name: modbus

ports:

- '502:502'

restart: unless-stopped

- Save. In the terminal (inside the docker folder) run:

docker-compose up -d

- Run docker ps — you should now see 5 containers, with modbus listed as Up.

1:30 – 5:00 PM  Secrets management and security documentation

- In the project root folder, create a file called .env (no file extension — just .env). Paste:

INFLUXDB_TOKEN=paste_your_actual_token_here

MQTT_ESP32_PASS=your_esp32_password

MQTT_NODERED_PASS=your_nodered_password

- Create a file called .gitignore in the project root. Paste:

.env

*.passwd

- Create a file called .env.example (safe to commit — shows structure without real values):

INFLUXDB_TOKEN=

MQTT_ESP32_PASS=

MQTT_NODERED_PASS=

- Create docs/cybersecurity-summary.md. Write these 4 sections: (1) MQTT Authentication — Mosquitto uses a password file, anonymous connections are disabled; (2) Topic ACLs — explain the acl.conf rules, which user can publish/subscribe to what; (3) Secrets Management — .env file holds credentials, .gitignore prevents it being committed to GitHub; (4) LWT Reliability — explain that the ESP32 pre-registers a 'OFFLINE' message with the broker, which fires automatically if the ESP32 disconnects unexpectedly.

- Create docs/mqtt-topic-hierarchy.md. Write a table with columns: Topic Path, Direction, Payload Description, Purpose. Include all 4 Sparkplug B topics. Below the table, explain the UNS structure: Namespace = spBv1.0, Group ID = conveyorLine, Message Type = DDATA/DCMD/NBIRTH/NDEATH, Edge Node ID = belt01, Device ID = beltDrive.

- Commit and push all new files.

Person 3  —  Node-RED & AI Engineer — Day 2

9:00 AM – 12:30 PM  Build the RUL estimator

- In Node-RED, drag a new function node onto the canvas. Connect it as a second output from your existing 'Format for InfluxDB' function node (drag from the same json node to this new function node, creating a parallel path).

- Double-click the new function node. Name it 'RUL Estimator'. Paste this code:

var score  = msg.payload[0].fields.anomaly_score;

var history = context.get('history') || [];

history.push(score);

if (history.length > 60) history.shift();

context.set('history', history);

if (history.length < 5) return null;

// Ordinary Least Squares linear regression

var n = history.length;

var sX=0, sY=0, sXY=0, sX2=0;

history.forEach(function(v,i){

sX+=i; sY+=v; sXY+=i*v; sX2+=i*i;

});

var slope = (n*sXY - sX*sY) / (n*sX2 - sX*sX);

var intercept = (sY - slope*sX) / n;

var rul = 9999;

if (slope > 0.0005) {

var steps = (1.0 - intercept) / slope;

var remaining = steps - (n - 1);

rul = Math.max(0, Math.round(remaining * 0.5 / 60)); // 0.5s per step → minutes

}

msg.payload = [{

measurement: 'rul_estimate',

fields: { rul_minutes: rul }

}];

return msg;

- Drag a second 'influxdb out' node. Same server config. Measurement: rul_estimate. Click Done.

- Connect: RUL Estimator function → second influxdb out.

- Click Deploy. After a few minutes of real data arriving, open Grafana and query the rul_estimate measurement — you should see values appearing.

1:30 – 3:00 PM  Build the relay control HTTP endpoint

- Drag an 'http in' node. Double-click: Method = POST, URL = /belt-control. Click Done.

- Drag a function node. Connect http in → function. Paste:

var cmd = msg.payload.relay_command || 'OFF';

msg.topic   = 'spBv1.0/conveyorLine/DCMD/belt01/beltDrive';

msg.payload = JSON.stringify({ relay_command: cmd });

return msg;

- Drag an 'mqtt out' node. Server: select your Mosquitto server. Leave Topic blank (msg.topic provides it). Click Done.

- Drag an 'http response' node. Connect http in → http response (to send back a 200 OK response).

- Connect: function → mqtt out. Click Deploy.

- Test from terminal:

curl -X POST http://localhost:1880/belt-control -H "Content-Type: application/json" -d "{\"relay_command\":\"ON\"}"

- You should hear the physical relay on Person 1's board click. This is the Digital Twin controlling the physical hardware. Run again with OFF to release it.

3:00 – 5:00 PM  Build simulation mode and Modbus feed

- Drag an 'inject' node. Set to repeat every 1 second. Drag a function node. Connect inject → function. Paste synthetic fault data:

msg.payload = [{

measurement: 'belt_vibration',

fields: { x:2.1, y:0.3, z:3.4, magnitude:4.0, anomaly_score:0.85,

fault_flag:1, relay_state:0 },

tags: { fault_type: 'roller_fault' }

}];

return msg;

- Drag a switch node. Property: flow.sim_mode. Rule 1: is true → output 1. Rule 2: otherwise → output 2.

- Wire: simulation function → switch input. Switch output 1 → influxdb out. Your real data function output → switch output 2 → influxdb out.

- Add toggle endpoint: drag http in (POST /sim-toggle), function (flow.set('sim_mode', !flow.get('sim_mode') || false); msg.payload={sim:flow.get('sim_mode')}; return msg;), http response. Click Deploy.

- Install Modbus: Manage Palette > Install > node-red-contrib-modbus. Add Modbus Read node polling the Modbus container at localhost:502 every 5 seconds. Connect to a debug node. This satisfies the industrial protocol requirement.

Person 4  —  Grafana & Documentation Lead — Day 2

9:00 AM – 12:30 PM  Build the full dashboard

- Open Grafana at http://localhost:3000. Open your 'Conveyor Belt Fault Detection' dashboard.

### Panel: Anomaly Score (Gauge)

- Add new panel. Visualization: Gauge. Flux query: filter for field anomaly_score from belt_vibration, last(). Min: 0, Max: 1. Thresholds: green 0, yellow 0.3 (#FFA500), red 0.7. Title: Fault Severity Score.

### Panel: RUL — Belt Life Remaining (Stat)

- Add panel. Visualization: Stat. Query rul_estimate measurement, field rul_minutes, last(). Unit: minutes. Title: Estimated Minutes to Fault.

### Panel: Fault Type (Text / Stat)

- Add panel. Visualization: Stat. Query belt_vibration, filter tag fault_type, last(). Title: Current Fault Type. Show the value as a string.

### Panel: X / Y / Z Raw Axes (Time Series)

- Add panel. Three queries (A, B, C) for fields x, y, z from belt_vibration over last 5 minutes. Title: Raw Accelerometer Data.

### Panel: Relay Status (State Timeline)

- Add panel. Visualization: State timeline. Query relay_state field from belt_vibration. Title: Belt Relay State (0=Running, 1=Stopped).

### Panel: Belt Stop / Resume Buttons

- Install Button Panel plugin: Administration > Plugins > search 'Button Panel' > Install.

- Add panel. Visualization: Button Panel. Add 3 buttons:

- STOP BELT — POST http://localhost:1880/belt-control — body: {"relay_command":"ON"}

- RESUME BELT — POST http://localhost:1880/belt-control — body: {"relay_command":"OFF"}

- SIMULATION MODE — POST http://localhost:1880/sim-toggle

- Click Apply. Save the dashboard.

1:30 – 5:00 PM  Wiring diagram + P&ID + report sections 3–5

- Open app.diagrams.net. Draw the ESP32-S3 wiring diagram: ESP32 board in the centre, MPU-6050 module on the left with 4 colour-coded wires (red=VCC→3.3V, black=GND, blue=SDA→GPIO8, yellow=SCL→GPIO9), relay module on the right (red=VCC→5V, black=GND, orange=IN→GPIO10). Label every GPIO number and pin name. Export as wiring-diagram.png. Save to docs/.

- Draw the P&ID in draw.io: conveyor belt as a horizontal rectangle labelled 'Conveyor Belt Drive', MPU-6050 as a diamond field instrument labelled 'VT-01 Belt Vibration', relay as a circle/valve symbol labelled 'XV-01 Belt Stop', ESP32+Node-RED as a square controller box labelled 'DCS / Edge Controller'. Add directional arrows. Export as pid-diagram.png. Save to docs/.

- In the shared Google Doc, write report sections 3 (Hardware: component specs table, wiring connections table, photo of your wired board, paste wiring diagram), 4 (MQTT: explain Sparkplug B UNS structure, paste the 4-topic table from Person 2's mqtt-topic-hierarchy.md, explain each topic), 5 (Edge ML: write out the Z-Score formula step by step, explain the 4 fault types and how each is detected, explain why Z-Score qualifies as edge AI).

- Commit and push all docs and diagrams.

✔  END-OF-DAY CHECKPOINT:  Multiple fault types (belt_slip, roller_fault, misalignment, belt_jam) visible in Grafana tagged columns | Physical relay clicks from STOP BELT button on Grafana dashboard | RUL panel shows minute estimate | Simulation mode injects synthetic data | MQTT ACLs active and tested | NTP timestamps in payload | All docs pushed to GitHub

DAY 3

Polish, Testing & Demo

GOAL: System fully reliable. Report 5–10 pages complete. All GitHub files committed. Demo rehearsed twice and ready.

Morning sync — 9:00 AM, 30 minutes. Run a complete team demo from scratch (restart Docker, reflash ESP32, show data in Grafana, trigger stop from dashboard). Write down every broken item. Fix broken things first — do not add new features.

Person 1  —  Hardware & Firmware Engineer — Day 3

9:00 – 11:00 AM  Reliability testing

- Test 1 — Disconnect/reconnect: unplug the USB cable from your laptop. Wait 15 seconds. Plug back in. The ESP32 should reconnect to WiFi and MQTT automatically and resume publishing within 20 seconds.

- Test 2 — Buffer flush: disconnect WiFi on your laptop for 10 seconds. Reconnect. Confirm buffered readings all arrive in InfluxDB at once (you will see a cluster of data points in Grafana).

- Test 3 — NTP: confirm Serial Monitor shows real ISO timestamps, not 1970-01-01.

- Test 4 — Auto-stop: go through the roller_fault phase (90–120 seconds elapsed). After 5 consecutive fault readings, the relay should click automatically and relay_state should show 1 in Grafana.

- Test 5 — Remote reset: while relay is ON, press RESUME BELT in Grafana. Confirm relay clicks off and relay_state returns to 0.

11:00 AM – 2:00 PM  Write demo script and rehearse

- Write a 2-minute spoken demo script with exactly these stages:

- Stage 1 — Normal (25s): Point to Grafana. 'This is the conveyor belt running normally. Vibration is low, anomaly score near zero, belt type shows normal.'

- Stage 2 — Belt slip (25s): Say 'We are now in the belt slip phase.' Show the fault_type tag changing. Gently shake the MPU-6050. 'Real sensor data confirms the slip signature.'

- Stage 3 — Roller fault and auto-stop (30s): 'Now we enter roller fault phase.' Tap the sensor rhythmically. After 5 readings — relay clicks. 'The edge AI on the ESP32 automatically stopped the belt to prevent damage.'

- Stage 4 — Remote restart (20s): Click RESUME BELT in Grafana. Relay releases. 'The operator remotely restarts the belt from the Digital Twin dashboard. Virtual controls physical hardware — that is the bidirectional Digital Twin.'

- Stage 5 — Simulation mode (20s): Click SIMULATION MODE toggle in Grafana. 'Simulation mode lets engineers test fault scenarios without the real machine. Watch the RUL estimate drop.'

- Rehearse this script as a team at least twice. Each person explains their own section.

Person 2  —  Infrastructure & MQTT Engineer — Day 3

9:00 – 11:00 AM  Final Docker and persistence tests

- Test persistence: run docker-compose down, then docker-compose up -d. Confirm Grafana dashboard is still there. Confirm InfluxDB data is still there. This proves volumes are working.

- Test .gitignore: run git status in the project root. Confirm the .env file does NOT appear in the list of files staged for commit.

- Run docker ps one final time. Screenshot it. You need 5 rows: mqtt, influxdb, nodered, grafana, modbus.

11:00 AM – 3:00 PM  Export flows, dashboard JSON, and final commit

- Export Node-RED flows: in Node-RED click the menu > Export > All Flows > Download. Save as flows.json in /nodered/. Push to GitHub.

- Export Grafana dashboard: in Grafana open the dashboard, click the gear icon (Dashboard settings) > JSON Model. Copy everything. Save as /grafana/grafana-dashboard.json. Push to GitHub.

- Verify your GitHub /docs/ folder contains all 6 files: architecture-diagram.png, wiring-diagram.png, pid-diagram.png, mqtt-topic-hierarchy.md, ml-model-description.md, cybersecurity-summary.md.

- Final push with everything: git add .  then  git commit -m 'Day 3: all final files'  then  git push

Person 3  —  Node-RED & AI Engineer — Day 3

9:00 AM – 12:30 PM  Finalise flows and write ML documentation

- Test every flow: MQTT→InfluxDB working (check Grafana), RUL updating (check rul_estimate measurement in Grafana), relay endpoint working (test with curl), simulation toggle working.

- Add error handling: drag a 'catch' node onto the canvas, connect to a 'debug' node. This catches silent errors that would otherwise fail silently.

- Write docs/ml-model-description.md with these sections: (1) Algorithm: Z-Score Statistical Anomaly Detection with multi-axis fault classification. (2) Formulas: write Z = (x − μ) / σ. For belt jam: magnitude < 0.2 × μ. For misalignment: |Y_zscore| > 3. (3) Calibration: 100 readings at startup, mean and std computed for both magnitude and Y-axis separately. (4) Fault types and thresholds: table of the 4 fault types, their Z-score ranges, and which axis triggers them. (5) Why this qualifies as edge AI: runs on the ESP32 microcontroller in under 1ms, no external library, no network connection required for inference. (6) RUL: explain OLS regression, write the slope formula, show a worked example with sample numbers.

1:30 – 3:00 PM  Write report sections 6 and 8

- Section 6 (Digital Twin): explain the 3-level Digital Twin maturity model — Digital Model (static architecture diagram), Digital Shadow (live sensor data on dashboard), Digital Twin (bidirectional: Grafana button clicks the physical relay). Show how bidirectional control flows: dashboard POST → Node-RED /belt-control → MQTT DCMD → ESP32 → GPIO 10 → relay. Include Grafana screenshot. Explain simulation mode.

- Section 8 (RUL Estimator): explain the OLS linear regression algorithm in your function node. Show a worked example: if anomaly_score increases from 0.1 to 0.4 over 10 readings, what is the slope? What does the RUL calculation give? Include a Grafana screenshot of the RUL panel.

Person 4  —  Grafana & Documentation Lead — Day 3

9:00 – 11:00 AM  Finalise dashboard

- Add an Alert rule: in Grafana, go to Alerting > Alert rules > New alert rule. Condition: when anomaly_score from belt_vibration IS ABOVE 0.7 FOR 30 seconds. Evaluation group: every 10 seconds. Add an annotation to the time series panel. Save. Trigger it by letting the simulation run — confirm a red annotation line appears.

- Polish: add units to all axes (m/s² for vibration, 0–1 for anomaly score, minutes for RUL). Make titles clear. Consistent colour theme.

- Screenshot the full dashboard with all panels visible. Save as dashboard-screenshot.png for the report.

11:00 AM – 2:00 PM  Complete the technical report

- Compile all sections from all team members into one Google Doc. Check all component names, topic strings, and GPIO numbers are consistent.

- Write Section 7 (Cybersecurity): MQTT auth (password file, allow_anonymous false), ACLs (which user can do what, paste the acl.conf content), secrets management (.env + .gitignore), LWT (explain the mechanism and how you tested it).

- Write Section 9 (Testing): create this table and fill in the Actual and Pass/Fail columns from real results:

| Test | Input | Expected | Actual | Pass/Fail |
| --- | --- | --- | --- | --- |
| Normal operation | ESP32 running 0–60s | fault_type=normal, score<0.1 in Grafana |  |  |
| Belt slip detection | Irregular shaking of MPU-6050 | fault_type=belt_slip, score 0.3–0.6 |  |  |
| Roller fault auto-stop | Rhythmic tapping for 5+ readings | relay clicks, fault_type=roller_fault, relay_state=1 |  |  |
| Remote belt restart | Click RESUME BELT in Grafana | Relay releases, relay_state=0 in Grafana |  |  |
| LWT offline detection | Unplug ESP32 USB | NDEATH topic receives OFFLINE within 10s |  |  |
| Simulation mode | Click SIMULATION MODE toggle | Grafana shows synthetic anomaly_score=0.85 |  |  |

- Write Section 10 (Conclusion): one paragraph per person on what they learned. One paragraph on what you would improve for a real production deployment (e.g. FFT frequency analysis, ML model trained on real belt data, cloud alerting via SMS).

- Add a title page, table of contents, all diagrams as appendices. File > Download > PDF. Save as CO326-Technical-Report.pdf in /report/. Push to GitHub.

2:00 – 4:00 PM  Presentation slides

- Open Google Slides. Create 12 slides:

- Slide 1: Title — Conveyor Belt Fault Detection, CO326, group number, 4 names

- Slide 2: Problem Statement — why conveyor belt faults are costly, what faults you detect

- Slide 3: System Architecture — paste architecture-diagram.png

- Slide 4: Hardware Setup — photo of wired ESP32 board + paste wiring-diagram.png

- Slide 5: MQTT Topic Hierarchy — table of all 4 Sparkplug B topics

- Slide 6: Edge ML — Z-Score formula, 4-fault-type table, why it runs on ESP32

- Slide 7: Digital Twin — 3-level maturity model diagram + Grafana screenshot

- Slide 8: Full Grafana Dashboard — large screenshot

- Slide 9: Cybersecurity Features — bullet list

- Slide 10: RUL Estimator — formula + sample calculation + Grafana RUL panel screenshot

- Slide 11: Testing Results — paste the 6-row test table

- Slide 12: Demo & Conclusion — 'see live demo' + one learning per person

- Export as PDF. Save as Presentation.pdf in /slides/. Push to GitHub.

FINAL TEAM REHEARSAL — 4:00 PM  Run the 2-minute demo script twice. Everyone explains their own component. Confirm all files are on GitHub. No last-minute uploads.

# SUBMISSION CHECKLIST

## GitHub folder structure

edge-ai-groupXX/

├── firmware/

│   ├── main.ino                ← full ESP32 firmware

│   └── README.md               ← pin wiring + libraries + how to flash

├── nodered/

│   └── flows.json              ← all exported Node-RED flows

├── docker/

│   ├── docker-compose.yml

│   └── mosquitto/

│       └── config/

│           ├── mosquitto.conf

│           └── acl.conf

├── docs/

│   ├── architecture-diagram.png

│   ├── wiring-diagram.png

│   ├── pid-diagram.png

│   ├── mqtt-topic-hierarchy.md

│   ├── ml-model-description.md

│   └── cybersecurity-summary.md

├── grafana/

│   └── grafana-dashboard.json

├── report/

│   └── CO326-Technical-Report.pdf

├── slides/

│   └── Presentation.pdf

├── .env.example

├── .gitignore

└── README.md

## Marks breakdown

| Criterion | Marks | Evidence |
| --- | --- | --- |
| System functionality | 25 | 4-layer pipeline live: physical ESP32 → MQTT → Node-RED → InfluxDB → Grafana. Relay physically actuates on fault. |
| Edge AI implementation | 20 | Z-Score multi-axis fault classifier on ESP32 hardware. 4 fault types detected. RUL OLS regression in Node-RED. ml-model-description.md. |
| MQTT communication | 15 | Sparkplug B UNS topics. Mosquitto auth + ACL. NBIRTH/NDEATH lifecycle. mqtt-topic-hierarchy.md. |
| Dashboard design | 10 | Grafana: vibration chart, fault severity gauge, RUL stat, fault type, relay timeline, belt stop buttons, alert annotation. |
| GitHub & code quality | 15 | Commits from Day 1, 2, 3 — not a single last upload. Clean folder structure. .gitignore. All 6 docs present. |
| Documentation | 10 | 5–10 page PDF report, all 10 sections, all 3 diagrams as appendices. cybersecurity-summary. README. |
| Innovation | 5 | Hardware implementation with real sensor. 4-fault-type classifier. Bidirectional Digital Twin. Simulation mode. Sparkplug B UNS. |

## Quick commands reference

| What you need to do | Command |
| --- | --- |
| Start all containers | cd docker then docker-compose up -d |
| Check containers are running | docker ps |
| Stop all containers | docker-compose down |
| Read a container's logs | docker logs -f nodered (or mqtt / influxdb / grafana) |
| Restart one container | docker restart mqtt |
| Open Node-RED in browser | http://localhost:1880 |
| Open InfluxDB in browser | http://localhost:8086 |
| Open Grafana in browser | http://localhost:3000 |
| Test relay ON from terminal | curl -X POST http://localhost:1880/belt-control -H "Content-Type: application/json" -d "{\"relay_command\":\"ON\"}" |
| Test relay OFF | curl -X POST http://localhost:1880/belt-control -H "Content-Type: application/json" -d "{\"relay_command\":\"OFF\"}" |
| Toggle simulation mode | curl -X POST http://localhost:1880/sim-toggle |
| Save and push code to GitHub | git add . then git commit -m "your message" then git push |

"A working system is good. A reliable, secure, and well-documented system is industrial-grade."

CO326  |  Department of Computer Engineering  |  Project #6 — Conveyor Belt Fault Detection