DEPARTMENT OF COMPUTER ENGINEERING

CO326: Industrial Internet of Things

Group Project Proposal  —  Project #6 from CO326 Project List

Conveyor Belt Fault Detection

Industrial Edge AI & Digital Twin — Hardware Implementation

Physical ESP32-S3  +  Real MPU-6050 Vibration Sensor  +  Relay Actuator  +  3-Day Build

| Field | Details |
| --- | --- |
| Course | CO326 — Industrial Internet of Things |
| Project Number | Project #6 from the CO326 35-project list |
| Project Type | Group Project (4 students) |
| Duration | 3 days (accelerated implementation) |
| Industry Domain | Manufacturing — Material Handling / Conveyor Systems |
| Physical Hardware | ESP32-S3 microcontroller (real device) |
| Sensor | MPU-6050 3-axis accelerometer (real physical sensor, I2C) |
| Actuator | 5V relay module (physically stops/restarts belt simulation) |
| Edge AI | Z-Score statistical anomaly detection + 4-fault classifier (runs on ESP32) |
| Cloud AI | OLS linear regression RUL estimator (runs in Node-RED) |
| Submission | Day 3 — Live demo + GitHub repository + technical report |

NOTE  This project is listed as Project #6 in the official CO326 35-project list: 'Simulate periodic vibration patterns and detect irregularities.' This proposal implements the full hardware version — real sensor data, real edge AI inference, real physical relay actuation — exceeding the simulated baseline.

# 1. Introduction

Conveyor belts are the backbone of modern manufacturing, warehousing, and logistics. A single undetected fault — a slipping belt, a seizing roller bearing, or a belt running off-track — can cause a complete production line stoppage, product damage, and injury risk. In high-volume facilities such as automotive assembly plants, distribution centres, and food processing lines, even 10 minutes of unplanned downtime can cost tens of thousands of dollars.

Conveyor belt faults have one property that makes them detectable early: they produce characteristic vibration signatures before catastrophic failure. A roller bearing beginning to seize creates periodic impact spikes at the roller rotation frequency. A belt slipping produces irregular amplitude bursts. A belt running misaligned causes asymmetric lateral vibration. These signatures can be detected at the edge — on a low-cost microcontroller attached directly to the belt drive — long before a full failure occurs.

This project implements a complete 4-layer Industrial IoT system for conveyor belt fault detection using a physical ESP32-S3 microcontroller and a physical MPU-6050 accelerometer. Edge AI inference (Z-Score anomaly detection with 4-fault classification) runs on the ESP32 hardware itself, requiring no cloud connection for detection. All data is published over a secure MQTT pipeline to a Grafana Digital Twin dashboard that enables real-time monitoring and remote belt stop/restart control.

# 2. Problem Statement

A conveyor belt in continuous operation exhibits well-defined vibration signatures for each fault type. This project detects and classifies four specific faults using the physical MPU-6050 accelerometer:

| Fault | Physical cause | Vibration signature | Detection method |
| --- | --- | --- | --- |
| Belt slip | Belt loses grip on drive roller — speed fluctuates | Irregular amplitude bursts, aperiodic spikes on X and Z axes | Magnitude Z-score 2–5σ above baseline, irregular burst pattern |
| Roller fault | Roller bearing begins seizing — rotates unevenly | Periodic impact spikes at ~1–2 Hz roller rotation frequency | Crest factor rises, repeating spike pattern, magnitude Z-score > 3σ |
| Belt misalignment | Belt drifts sideways off rollers | Sustained lateral (Y-axis) vibration increase without X/Z change | Y-axis Z-score > 3σ while magnitude Z-score remains low |
| Belt jam / stall | Belt caught on obstruction or drive motor stalls | Magnitude collapses to near-zero (stall) OR single large spike (snag) | Magnitude < 20% of baseline mean (stall) OR Z-score > 8σ (snag) |

All four faults are detectable using only the MPU-6050 accelerometer — no additional sensors are required. The physical board, when tapped, shaken, tilted, or held still, reproduces each fault signature authentically, making the hardware demonstration fully representative of a real belt installation.

# 3. Project Objectives

- Design and implement a 4-layer Industrial IoT architecture: Perception, Transport, Edge-Logic, Application

- Deploy physical hardware — ESP32-S3 + MPU-6050 + relay — and collect real vibration data

- Implement edge AI on the physical ESP32-S3: Z-Score anomaly detection classifying 4 fault types in real time

- Build a secure MQTT pipeline using Mosquitto with authentication, ACLs, and Sparkplug B topic structure

- Develop a true bidirectional Digital Twin: Grafana dashboard commands physically actuate the relay module

- Implement RUL estimation in Node-RED using OLS linear regression on the anomaly score trend

- Produce all required CO326 deliverables: GitHub repo, report (5–10 pages), diagrams, live demo

# 4. System Architecture — 4-Layer IIoT

## 4.1 Layer 1 — Perception Layer (ESP32-S3 physical hardware)

The ESP32-S3 is the physical edge node. A real MPU-6050 accelerometer is wired to the board and reads genuine vibration data at 200 Hz. Edge AI inference runs on the microcontroller itself — anomaly scoring happens before any data leaves the device.

| Component | Specification | Function in this project |
| --- | --- | --- |
| ESP32-S3 Dev Board | Dual-core Xtensa LX7 @ 240 MHz, WiFi, 8MB Flash | Edge node: runs firmware, WiFi, MQTT, and edge ML inference |
| MPU-6050 (physical) | 6-axis IMU, I2C bus, ±8g range, 16-bit ADC, up to 1kHz | Real vibration measurement — wired via SDA=GPIO8, SCL=GPIO9 |
| Relay Module (physical) | 5V single channel, optocoupler isolated, 10A/250VAC rated | Physical belt stop actuator — controlled via GPIO10 |
| Edge AI algorithm | Z-Score with 4-fault classifier — runs on ESP32 in <1ms | Classifies belt_slip, roller_fault, misalignment, belt_jam |
| Fault simulation | 4-phase firmware simulation on top of real sensor readings | Reproduces all 4 fault types reliably for demo purposes |

## 4.2 Layer 2 — Transport Layer (MQTT + Sparkplug B)

| Topic | Direction | Payload | Purpose |
| --- | --- | --- | --- |
| spBv1.0/conveyorLine/NBIRTH/belt01 | ESP32 → Broker | {"status":"ONLINE"} | Device birth certificate on connect |
| spBv1.0/conveyorLine/DDATA/belt01/beltDrive | ESP32 → Broker | x, y, z, magnitude, anomaly_score, fault_flag, fault_type, relay_state, timestamp | Live sensor + AI data stream every 500ms |
| spBv1.0/conveyorLine/DCMD/belt01/beltDrive | Broker → ESP32 | {"relay_command":"ON"¦"OFF"} | Remote belt stop/restart command from dashboard |
| spBv1.0/conveyorLine/NDEATH/belt01 | ESP32 → Broker (LWT) | OFFLINE | Auto-published on unexpected disconnect |

## 4.3 Layer 3 — Edge-Logic Layer (Node-RED)

- Subscribe to all DDATA topics via authenticated Mosquitto connection

- Parse JSON payload: extract x, y, z, magnitude, anomaly_score, fault_type, relay_state

- Format data for InfluxDB: write belt_vibration measurement with fault_type as a tag for filtering

- RUL Estimator: OLS linear regression over rolling 60-sample anomaly score window — projects time to score = 1.0

- Rule engine: if anomaly_score > 0.7 sustained for 5 consecutive readings, publish relay ON command automatically

- Bidirectional control: HTTP endpoint /belt-control receives POST from Grafana dashboard, publishes DCMD topic

- Simulation mode: HTTP endpoint /sim-toggle switches between real sensor data and synthetic fault data

- Modbus TCP: polls simulated Modbus server for industrial protocol compliance demonstration

## 4.4 Layer 4 — Application Layer (InfluxDB + Grafana)

- InfluxDB 2.x stores all time-series metrics with fault_type as a filterable tag

- Grafana SCADA dashboard: live vibration chart, fault severity gauge, fault type display, RUL countdown, relay state timeline

- Digital Twin panel: virtual belt state mirrors physical relay state in real time

- Bidirectional control: STOP BELT button → Node-RED → MQTT DCMD → ESP32 → relay physically activates

- What-If simulation mode: inject synthetic fault data and observe RUL response without physical sensor

- Alert rule: fires when anomaly_score > 0.7 sustained for 30 seconds, appears as red annotation on timeline

# 5. Hardware Design

## 5.1 Component List

| Component | Specification | Qty | Role |
| --- | --- | --- | --- |
| ESP32-S3 Dev Board | Dual-core LX7 @ 240 MHz, WiFi, 512KB SRAM, 8MB Flash | 1 | Edge processing node |
| MPU-6050 Module | 6-axis IMU, I2C, ±8g, 16-bit ADC, 200Hz sampling | 1 | Real physical vibration sensor |
| 5V Relay Module | Optocoupler isolated, 10A/250VAC, active-LOW control | 1 | Belt stop actuator |
| USB-C / micro-USB cable | Matching the ESP32-S3 port type | 1 | Power + programming |
| Breadboard | Full-size 830 tie-point | 1 | Prototyping |
| Jumper wires | Male-to-female, 20cm | 10 | Connections |

## 5.2 Pin Wiring — MPU-6050 to ESP32-S3

| MPU-6050 Pin | ESP32-S3 Pin | Wire colour | Critical note |
| --- | --- | --- | --- |
| VCC | 3.3V | Red | MUST be 3.3V — 5V destroys ESP32 I2C pins |
| GND | GND | Black | Common ground |
| SDA | GPIO 8 | Blue | I2C data |
| SCL | GPIO 9 | Yellow | I2C clock |

## 5.3 Pin Wiring — Relay to ESP32-S3

| Relay Pin | ESP32-S3 Pin | Wire colour | Critical note |
| --- | --- | --- | --- |
| VCC | 5V (VBUS) | Red | Relay needs 5V — 3.3V is insufficient |
| GND | GND | Black | Share with MPU-6050 GND |
| IN | GPIO 10 | Orange | Active LOW — LOW = relay energised on most modules |

## 5.4 Fault Simulation Strategy

The ESP32-S3 firmware runs a 4-phase simulation cycle on top of real MPU-6050 readings. The real sensor provides authentic baseline vibration; the firmware adds fault-specific modifications at known time intervals to guarantee each fault type is demonstrated reliably.

| Phase | Elapsed time | Fault type | What the firmware adds to real sensor data | Expected anomaly_score |
| --- | --- | --- | --- | --- |
| Normal | 0–60 seconds | none | No modification — real sensor baseline only | 0.0–0.15 |
| Belt slip | 60–90 seconds | belt_slip | Random amplitude bursts on X and Z (30% probability per reading) | 0.3–0.6 |
| Roller fault | 90–120 seconds | roller_fault | Periodic +1.5g spike on X every 4th reading (simulates ~1Hz roller impact) | 0.5–0.85 |
| Misalignment | 120–150 seconds | misalignment | Sustained +1.8g offset added to Y axis | Y-axis Z-score > 3, magnitude score 0.3–0.7 |
| Belt jam | 150–165 seconds | belt_jam | All axis values multiplied by 0.05 — near-zero magnitude (stall) | 0.9–1.0 |

Physical interaction with the MPU-6050 (tapping, shaking, tilting, holding still) during any phase produces additional genuine anomaly events on top of the simulated pattern. This means every demo includes a mix of real and simulated fault data, directly satisfying the hardware implementation requirement.

# 6. Edge AI Design

## 6.1 Z-Score Fault Detection (on ESP32-S3 hardware)

The Z-Score algorithm detects anomalies by measuring how far the current vibration magnitude deviates from the calibrated baseline. It runs entirely on the ESP32-S3 in under 1 millisecond and requires no external library. Three independent checks run in parallel on every reading:

| Check | Formula | Detects | Threshold |
| --- | --- | --- | --- |
| Magnitude Z-score | z = (magnitude − μ) / σ where μ and σ from 100-sample calibration | Belt slip and roller fault — magnitude rises above baseline | z > 3.0 triggers fault_flag |
| Y-axis Z-score | z_y = ¦y − μ_y¦ / σ_y (separate Y-axis calibration) | Belt misalignment — Y drifts sideways without magnitude change | z_y > 3.0 triggers misalignment fault type |
| Stall detection | stall = magnitude < 0.2 × μ (magnitude collapses) | Belt jam / stall — belt stops moving, vibration near-zero | Magnitude < 20% of baseline mean |

Anomaly score is normalised to 0.0–1.0: stall → 0.95, misalignment → min(z_y / 10, 0.9), all others → min(z_mag / 10, 1.0). The fault_type string label is determined by priority order: belt_jam > misalignment > roller_fault (score > 0.6) > belt_slip (score > 0.3) > normal.

## 6.2 RUL Estimation (Node-RED — Layer 3)

- Algorithm: Ordinary Least Squares (OLS) linear regression implemented as a Node-RED function node

- Input: rolling window of last 60 anomaly_score values (30 seconds of data at 2Hz)

- Slope formula: slope = (n·ΣxᵢYᵢ − ΣxᵢΣYᵢ) / (n·Σxᵢ² − (Σxᵢ)²) where xᵢ is sample index, Yᵢ is anomaly_score

- RUL formula: RUL_minutes = (1.0 − current_score) / max(slope, 0.0005) × 0.5 / 60

- Recomputed on every new data point. Written to rul_estimate measurement in InfluxDB for Grafana display

- Alert fires in Grafana when RUL < 5 minutes

# 7. Digital Twin Design

| Twin Capability | Requirement | Implementation |
| --- | --- | --- |
| Digital Model | Static representation | Architecture diagram and P&ID in docs/ folder and report appendices |
| Digital Shadow | One-way data sync | Live Grafana dashboard: vibration waveform, anomaly score, fault type, relay state — all from physical ESP32 |
| Digital Twin (required) | Bidirectional sync | STOP BELT button in Grafana → POST to Node-RED → MQTT DCMD published → ESP32 GPIO 10 → relay physically activates |
| Simulation Mode | What-If scenarios | POST to /sim-toggle endpoint in Node-RED switches data source to synthetic fault injection — RUL collapses visibly |
| State Consistency | Mismatch detection | Node-RED compares commanded relay state vs reported relay_state field; logs mismatch after 5 seconds |

# 8. MQTT Unified Namespace (UNS) Design

| UNS Level | Value | Description |
| --- | --- | --- |
| Namespace | spBv1.0 | Sparkplug B specification version 1.0 |
| Group ID | conveyorLine | Production line / facility identifier |
| Message Type | DDATA / DCMD / NBIRTH / NDEATH | Sparkplug B message class |
| Edge Node ID | belt01 | ESP32-S3 physical node identifier |
| Device ID | beltDrive | Conveyor belt drive asset identifier |

# 9. Cybersecurity & Reliability

## 9.1 Security Controls

- Mosquitto configured with username/password authentication — anonymous connections disabled

- Topic ACLs: esp32user may only publish DDATA/NBIRTH/NDEATH and subscribe to DCMD; nodered has full access

- Credentials stored in .env file — never hardcoded in source files or committed to GitHub

- .gitignore prevents .env from being uploaded to the public GitHub repository

## 9.2 Reliability Controls

- MQTT LWT: NDEATH message pre-registered — auto-published when ESP32 disconnects unexpectedly

- Circular buffer: 50 readings stored in ESP32 SRAM during WiFi outages — flushed on reconnect

- Auto-reconnect: firmware retries MQTT connection every 3 seconds when disconnected

- NTP timestamp: all payloads include ISO 8601 UTC timestamp from pool.ntp.org

# 10. Technology Stack

| Layer | Technology | Purpose |
| --- | --- | --- |
| Edge Device | ESP32-S3 (physical) | Firmware, real sensor reading, Z-Score ML, MQTT publish, relay control |
| Sensor | MPU-6050 (physical, I2C) | Real 3-axis vibration at 200Hz — wired via GPIO 8/9 |
| Actuator | 5V Relay (physical) | Physical belt stop — controlled via GPIO 10 |
| Broker | Mosquitto 2.0 (Docker) | MQTT broker with password auth + ACL file |
| Flow Engine | Node-RED (Docker) | Data routing, RUL regression, relay HTTP endpoint, simulation mode |
| Database | InfluxDB 2.7 (Docker) | Time-series storage with fault_type tags |
| Dashboard | Grafana (Docker) | SCADA dashboard, Digital Twin, bidirectional belt controls, alerts |
| Protocol | MQTT + Sparkplug B v1.0 | Structured UNS industrial messaging |
| Secondary Protocol | Modbus TCP (Docker, simulated) | Industrial protocol compliance |
| Deployment | Docker Compose | All server components in one docker-compose up command |
| Version Control | GitHub | Firmware, flows, config, docs — continuous commits required |

# 11. 3-Day Implementation Plan

| Day | Tasks | End-of-day target |
| --- | --- | --- |
| Day 1 — Foundation | GitHub repo + Docker stack (4 containers) + MQTT auth + InfluxDB setup + physical ESP32 wiring + firmware upload + Node-RED MQTT→InfluxDB flow + first Grafana chart | All 4 containers running. Physical ESP32 sending real MPU-6050 data. Live Grafana chart updating. |
| Day 2 — Intelligence | Verify fault phases on hardware + NTP timestamps + circular buffer + MQTT ACLs + Modbus container + Node-RED RUL estimator + relay control endpoint + simulation mode + full Grafana dashboard with buttons | 4 fault types visible in Grafana. Physical relay clicks from dashboard. RUL live. MQTT security tested. |
| Day 3 — Polish & Demo | Hardware reliability tests + demo script + firmware README + Docker persistence test + export flows + complete report (5–10 pages) + all diagrams + presentation slides + final push | Full system reliable. All files on GitHub. Report complete. Demo rehearsed twice. |

# 12. Deliverables

| Deliverable | Format / Location | Owner |
| --- | --- | --- |
| Live system demo | Day 3 — all 4 layers live, physical relay actuating on fault | All |
| ESP32-S3 firmware | GitHub /firmware/main.ino + README.md | Person 1 |
| Node-RED flows | GitHub /nodered/flows.json | Person 3 |
| Docker config | GitHub /docker/docker-compose.yml + mosquitto files | Person 2 |
| System architecture diagram | GitHub /docs/architecture-diagram.png + report Appendix A | Person 4 |
| Electrical wiring diagram | GitHub /docs/wiring-diagram.png + report Appendix B | Person 4 |
| Simplified P&ID | GitHub /docs/pid-diagram.png + report Appendix C | Person 2 |
| MQTT topic hierarchy | GitHub /docs/mqtt-topic-hierarchy.md | Person 2 |
| ML model description | GitHub /docs/ml-model-description.md | Person 3 |
| Cybersecurity summary | GitHub /docs/cybersecurity-summary.md | Person 2 |
| Grafana dashboard JSON | GitHub /grafana/grafana-dashboard.json | Person 4 |
| Technical report (5–10 pages) | GitHub /report/ + LMS submission | Person 4 |
| Presentation slides | GitHub /slides/ + LMS submission | Person 4 |

# 13. Assessment Criteria Mapping

| Criterion | Marks | How this project addresses it |
| --- | --- | --- |
| System functionality | 25 | Full 4-layer pipeline: physical ESP32-S3 → MQTT Sparkplug B → Node-RED → InfluxDB → Grafana, end-to-end with real vibration data. Relay physically actuates on fault detection. |
| Edge AI implementation | 20 | Z-Score 4-fault classifier running on physical ESP32 hardware — no external library, <1ms inference. OLS linear regression RUL estimator in Node-RED. ml-model-description.md documents both. |
| MQTT communication | 15 | Sparkplug B UNS with DDATA/DCMD/NBIRTH/NDEATH. Mosquitto with password auth and topic ACLs. mqtt-topic-hierarchy.md documents full structure. |
| Dashboard design | 10 | Grafana SCADA dashboard: live vibration waveform, fault severity gauge (4 colours), RUL countdown, fault type text, relay state timeline, STOP/RESUME buttons, alert annotation. |
| GitHub & code quality | 15 | Commits from Day 1, 2, and 3 — not a single upload. Clean folder structure. .env in .gitignore. All 6 docs committed with meaningful commit messages. |
| Documentation | 10 | 5–10 page technical report with 10 sections + all diagrams + testing table. cybersecurity-summary.md. README.md with full run instructions. |
| Innovation | 5 | Hardware implementation (real sensor + physical relay). 4-fault type classifier beyond simple threshold. Bidirectional Digital Twin. Sparkplug B UNS. Fault_type tags in InfluxDB. |

"Simulate periodic vibration patterns and detect irregularities." — CO326 Project #6 specification

This proposal implements the full hardware version of that specification.

End of Project Proposal  |  CO326  |  Department of Computer Engineering